SPECIMEN CALCULATIONS
For IF,SM = 0.6 A

- Considering set of readings for No load case from Table 1.

Input power to the motor = W1+W2
                                                   =(26+60) W
                                                   = 86 W

Power factor of the motor = Active powerApparent power
= W1+W23×VL×IL
= 86 W3×400×A1
= 86 W3×400×0.24
                                                     = 0.517

Armature resistance loss of DC motor (1) = A32×Ra
= 0×15 Ω
= 0 W

Output power of DC motor (2) = Va×Ia
= V2×A3
= 200 V×0
= 0 W

Output power of Synchronous motor = (1)+(2)+Rotational loss
= 0+0+30 W
= 30 W

Efficiency of Synchronous motor = PoutPin×100%
= 30 W86 W×100%
                                                                 = 34.88 %

For IF,SM = 0.6 A

- Considering set of readings for Bulb load 1 from Table 1.

Input power to the motor = W1+W2
                                                   =(58+75) W
                                                    = 133 W

Power factor of the motor = Active powerApparent power
= W1+W23×VL×IL
= 133 W3×400×A1
= 133 W3×400×0.30
                                                     = 0.640

Armature resistance loss of DC motor (1) = A32×Ra
                                                                                  = 0.24 ×0.24 ×15 Ω
                                                                                  = 0.864 W

Output power of DC motor (2) = Va×Ia
= V2×A3
                                                             = 196 V×0.24A
                                                              = 47.04W

Output power of Synchronous motor = (1)+(2)+Rotational loss
= 0.864+47.04+30 W
= 77.904 W

Efficiency of Synchronous motor = PoutPin×100%
                                                                 = 77.904 W133 W×100%
= 58.57 %

For IF,SM = 0.6 A

- Considering set of readings for Bulb load 2 from Table 1.

Input power to the motor = W1+W2
=(96+105) W
                                                   = 201 W

Power factor of the motor = Active powerApparent power
= W1+W23×VL×IL
= 201W3×400×A1
= 201 W3×400×0.40
                                                    = 0.725

Armature resistance loss of DC motor (1) = A32×Ra
                                                                                 = 0.6 ×0.6 ×15 Ω
= 5.4 W

Output power of DC motor (2) = Va×Ia
= V2×A3
                                                             = 188 V×0.6A
                                                             = 112.8 W

Output power of Synchronous motor = (1)+(2)+Rotational loss
                                                                         = 5.4+112.8+30 W
                                                                         = 148.2 W

Efficiency of Synchronous motor = PoutPin×100%
                                                                 = 148.2 W201 W×100%
                                                                 = 73.73 %

For IF,SM = 0.6 A

- Considering set of readings for Bulb load 3 from Table 1.

Input power to the motor = W1+W2
                                                    =(128+140) W
                                                    = 268 W

Power factor of the motor = Active powerApparent power
= W1+W23×VL×IL
= 268 W3×400×A1
= 268 W3×400×0.54
                                                    = 0.716

Armature resistance loss of DC motor (1) = A32×Ra
                                                                                 = 0.94 ×0.94 ×15 Ω
= 13.254 W

Output power of DC motor (2) = Va×Ia
= V2×A3
                                                             = 178 V×0.94A
                                                             = 167.32 W

Output power of Synchronous motor = (1)+(2)+Rotational loss
                                                                          = 13.254+167.32+30 W
                                                                          = 210.574 W

Efficiency of Synchronous motor = PoutPin×100%
                                                                 = 210.574 W268 W×100%
                                                                 = 78.57 %